室内壁挂天线

Indoor hanging antenna

| 说 明 书 SPECIFICATION Model | TDJ-800/2500-7/8 |
| --- | --- |
| Electrical Specifications |  |
| Frequency Range 800-960/1710-2700MHZ VSWR ≤1.4 Input Impedance 50Ω Gain 7/8dBi Polarization Vertical Horizontal Beamwidth 90°/ 75° Vertical Beamwidth 65° Electrical Downtilt 0° Front to Back Ratio ≥23dB Max,power 50W Connector Type N Female Intermediation IM3 ≤-107dBm Lightning Protection DC ground |  |
| Mechanical Specifications |  |
| Dimension 210×180×44mm Weight 0.6kg Working Temperature -40℃~+70℃ Radome material UV Protected ABS |  |

| 型 号 | TDJ-800/2500-7/8 |
| --- | --- |
| 电 性 能 指 标 |  |
| 频率范围 800-960/1710-2700MHZ 电压驻波比 ≤1.4 输入阻抗 50Ω 增益 7/8dBi 极化形式 垂直极化 水平波瓣宽度 90°/ 75° 垂直波瓣宽度 65° 电子下倾角 0° 前后比 ≥23dB 最大功率 50W 接口形式 N型阴头 三阶互调 ≤-107dBm 闪电保护 直流接地 |  |
| 机 械 指 标 |  |
| 尺寸 210×180×44mm 重量 0.6kg 工作温度 -40℃~+70℃ 天线罩材料 防紫外线ABS |  |